4X4 Fused PM Fiber Splitter Module

Features Applications

Wavelength 532 - 2100nm available Coherent Communication

Low Insertion Loss Power Monitor

High Stability & Reliability Fiber Sensor

Specifications

Parameter Unit Value

Coupling Ratio % 25:25:25:25

Type - 4x4

Center Wavelength nm 1550Operating Bandwidth nm ±15

Max. Excess Loss dB 1.2

Insertion Loss 7.5

Min. Extinction Ratio dB 20

Min. Directivity dB 50

Max. Return Loss dB 50

Max. Optical Power (CW) mW 300

Fiber Type - PM1550 fiber, 900mm Loose Tube

Package Dimensions mm 100*80*10

Operating Temperature ℃ -5~+75

Storage Temperature ℃ -40~+85

With connectors, the handling power is 50mW only, IL is 0.3dB higher, RL is 5dB lower, and ER is 2dB lower.Connector key is aligned to slow axis.

Package Dimensions

Ordering Information

PMFUC-①①①①-②②-③③③-④-⑤-⑥-⑦-⑧

①①①① - Wavelength: 1550=1550nm, SSSS=Specified

②② - Port Type: 44=4x4

③③③ - Fiber Type: 001=PM1550 fiber

④ - Package Dimensions: 0=108*80*10、 S=Specified

⑤ - Pigtail Type: 0=bare fiber, 1=900μm loose tube 2=2.0mm loose tube 3=3.0mm loose tube

⑥ - Fiber Length: 0.8=0.8m, 1.0=1m, S=Specified

⑦ - Connector Type for 0=FC/UPC, 1=FC/APC, 2=SC/UPC, 3=SC/APC, N=None, S=Specified

⑧ - Connector Type for 0=FC/UPC, 1=FC/APC, 2=SC/UPC, 3=SC/APC, N=None, S=Specified