1x2(2x2) PM Filter Coupler

1. Features

- Low Insertion Loss

- High Extinction Ratio

- High Isolation

- High Stability and Reliability

2. Applications

- EDFA

- Fiber Optical Instrument

- Power Monitoring

- Fiber Sensor

- Specifications

| Parameters | Unit | Value |
| --- | --- | --- |
| Operating Wavelength | nm | 1310 |
| Operating Bandwidth | nm | ±30 |
| Insertion Loss | dB | ≤3.6 |
| Extinction Ratio | dB | ≥20 |
| Return Loss | dB | ≥50 |
| Directionality | dB | ≥50 |
| Power | mW | 300 |
| Operating Temperature | ℃ | -5~+65 |
| Storage Temperature | ℃ | -40～+80 |
| Port Type | / | 1x2 |
| Fiber Type | / | PM 13-U25D |
| Fiber Length | m | 1.0±0.1m, others on request |
| Connector | / | FC/APC, others on request |
| Package Dimension | mm | Φ5.5x38 (900μm Loose Tube ) |

For device with connector, IL is 0.3dB higher, RL is 5dB lower, ER is 2dB lower.

The default connector key is aligned to slow axis.

4．Package Dimension

5. Ordering information

| Couper | - | XXX | XXX | XX | XXX | XX | XX | - | X | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Port NO | Wavelength | Splitting Ratio | Modules type | Fiber Type | Fiber Length |  | Com connector | Port Connector |
| PM Filter Coupler =PMFCP |  | 12=1X2 | 131=1310nm | 50=50/50 | ST1=Ф3.0X45 | 02=250um fiber | 10=1.0m |  | 0=None | 0=None |
|  |  | 22=2X2 | 148=1480nm | 90=90/10 | ST2=Ф3.0X54 | 09=0.9mm Tube | XX=Customized |  | 1=FC/UPC | 1=FC/UPC |
|  |  |  | 155=1550nm | 99=99/01 | SB=90 X20 X10 | 20=2.0mm Tube |  |  | 2=FC/APC | 2=FC/APC |
|  |  |  | 980=980nm | 95=95/5 | XX=Customized | 30=3.0mm Tube |  |  | 3=SC/UPC | 3=SC/UPC |
|  |  |  | 106=1064nm |  |  | XX=Customized |  |  | 4=SC/APC | 4=SC/APC |
|  |  |  | 780=780nm |  |  |  |  |  | 5=LC/UPC | 5=LC/UPC |
|  |  |  |  |  |  |  |  |  | 6=LC/APC | 6=LC/APC |
|  |  |  |  |  |  |  |  |  | X=Customized | X=Customized |

| Part Number | Product Description |
| --- | --- |
| PMFCP-1213195ST10910-22(S) | 1*2 PM Filter Coupler, Splitting Ratio 95/5, Steel Tube type, PM 13-U25D fiber With 0.9mm Hytrel White Tube, fiber Length 1m (with connector), connector FC/APC, connector key is aligned to slow axis, fast axis blocked. |