1x2 Filter PM Fiber Standard Coupler

1. Features

- Low Insertion Loss

- Low Extinction Ratio

- Available for Slow Only or Fast Axis App

- High Stability and Reliability

2. Applications

- EDFA

- Fiber Optical Instrument

- Coherent Communication

- Power Monitoring

- Fiber Sensor

3. Specifications

| Parameters |  | Unit | 1 x 2 |
| --- | --- | --- | --- |
| Splitting Ratio |  | nm | 50/50 |
| Operating Wavelength |  | nm | 1310/1550 |
| Operating Bandwidth |  | nm | ±40 |
| Insertion loss |  | nm | ≤3.8 |
| Extinction Ratio |  | dB | ≥20 |
| Return Loss |  | dB | ≥50 |
| Power Handling |  | mW | ≤500 |
| Operating Temperature |  | ℃ | -10~+65 |
| Storage Temperature |  | ℃ | -40～+85 |
| Fiber Type |  | / | Corning Panda fiber |
| Fiber Length |  | m | 1.0±0.1m， others on request |
| Connector |  | / | FC/APC,FC/UPC， others on request |
| Package Dimension | 250μm Bare Fiber | mm | φ3.0×35 |
|  | 900μm Loose Tube | mm | φ3.0×38 |
|  | 2mm/3mm Loose Tube | mm | 90×20×10 |

| Splitting Ratio | Unit | Insertion loss |
| --- | --- | --- |
|  |  | 1 x 2 |
| 50/50 | dB | ≤3.8 |
| 20/80 | dB | ≤8.2/1.7 |
| 10/90 | dB | ≤11.8/1.0 |
| 5/95 | dB | ≤14.9/0.9 |
| 1/99 | dB | ≤22.2/0.6 |

*Above specifications are for devices with the connectors.

*The PM fiber and the connector key are aligned to the slow axis, fast axis is blocked

4．Package Dimension

5. Ordering information

| Couper | - | XXX | XXX | XX | XXX | XX | XX | - | X | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Port NO | Wavelength | Splitting Ratio | Modules type | Fiber Type | Fiber Length |  | Com connector | Port Connector |
| PMC= PM Coupler |  | 12=1X2 | 131=1310nm | 50=50/50 | ST1=Ф5.5X35 | 02=250um fiber | 10=1.0m |  | 0=None | 0=None |
|  |  | 22=2X2 | 155=1550nm | 10=10/90 | ST2=Ф5.5X38 | 09=0.9mm Tube | XX=Customized |  | 1=FC/UPC | 1=FC/UPC |
|  |  |  | 980=980nm | 01=01/99 | SB=90 X20 X10 | 20=2.0mm Tube |  |  | 2=FC/APC | 2=FC/APC |
|  |  |  | 106=1064nm |  | XX=Customized | 30=3.0mm Tube |  |  | 3=SC/UPC | 3=SC/UPC |
|  |  |  | 780=780nm |  |  | XX=Customized |  |  | 4=SC/APC | 4=SC/APC |
|  |  |  |  |  |  |  |  |  | 5=LC/UPC | 5=LC/UPC |
|  |  |  |  |  |  |  |  |  | 6=LC/APC | 6=LC/APC |
|  |  |  |  |  |  |  |  |  | X=Customized | X=Customized |

| Part Number | Product Description |
| --- | --- |
| PMC-1215510ST20910-22(S) | 1*2 Filter PM Fiber Coupler，Splitting Ratio 50/50,Steel Tube type, PM 1550nm Panda fiber With 0.9mm Hytrel Blue Tube, fiber Length 1m (with connector)， connector FC/APC， connector key is aligned to slow axis. |