(2x2) Filter PM Fiber Standard Coupler

- Features

- Low Insertion Loss

- Low Extinction Ratio

- Available for Slow Only or Fast Axis App

- High Stability and Reliability

- Applications

- EDFA

- Fiber Optical Instrument

- Coherent Communication

- Power Monitoring

- Fiber Sensor

- Specifications

| Parameters | Unit | Value |
| --- | --- | --- |
| Operating Wavelength | nm | 1550 |
| Operating Bandwidth | nm | +/-40 |
| Splitting Ratio | dB | 10/90 |
| Insertion loss | dB | ≤11.3/1.0 |
| Typical Excess Loss | dB | ≤0.7 |
| Uniformity | dB | ≤0.5 |
| Polarization Extinction Ratio | dB | ≥22 |
| Return Loss | dB | ≥55 |
| Power Handling | mW | ≤300 |

| Directivity |  | dB | ≥55 |
| --- | --- | --- | --- |
| Operating Temperature |  | ℃ | -5~+65 |
| Storage Temperature |  | ℃ | -40～+80 |
| Port Type |  | / | 2x2; |
| Fiber Type |  | / | Corning Panda fiber |
| Fiber Length |  | m | 1.0±0.05 |
| Connector |  | / | FC/APC |
| Package Dimension | 900μm Loose Tube | mm | Φ5.5×38 |

For device with connector, IL is 0.3dB higher, RL is 5dB lower, ER is 2dB lower. The default connector key is aligned to slow axis.

- Package Dimension

- Ordering information

| Coup er | - | XXX | XXX | XX | XXX | XX | XX | - | X | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Port NO | Wavelength | Splitting Ratio | Modules type | Fiber Type | Fiber Length |  | Com connector | Port Connector |
| PMC= PM Coupler |  | 12=1X2 | 131=1310nm | 50=50/50 | ST1=Ф5.5X35 | 02=250um fiber | 10=1.0m |  | 0=None | 0=None |
|  |  | 22=2X2 | 155=1550nm | 10=10/90 | ST2=Ф5.5X38 | 09=0.9mm Tube | XX=Customized |  | 1=FC/UPC | 1=FC/UPC |
|  |  |  | 980=980nm | 01=01/99 | SB=90 X20 X10 | 20=2.0mm Tube |  |  | 2=FC/APC | 2=FC/APC |
|  |  |  | 106=1064nm |  | XX=Customized | 30=3.0mm Tube |  |  | 3=SC/UPC | 3=SC/UPC |
|  |  |  | 780=780nm |  |  | XX=Customized |  |  | 4=SC/APC | 4=SC/APC |
|  |  |  |  |  |  |  |  |  | 5=LC/UPC | 5=LC/UPC |
|  |  |  |  |  |  |  |  |  | 6=LC/APC | 6=LC/APC |
|  |  |  |  |  |  |  |  |  | X=Customized | X=Customized |

| Part Number | Product Description |
| --- | --- |
| PMC-2215510ST20910-22(S) | 2*2 Filter PM Fiber Coupler，Splitting Ratio 10/90, Steel Tube type, PM 1550nm Panda fiber With 0.9mm Hytrel White Tube, fiber Length 1m (with connector)， connector FC/APC， connector key is aligned to slow axis. |