In-Line Polarizer

1 Features

- Low Insertion Loss

- High Extinction Ratio & High Isolation

- High stability and reliability

- Telcordia Compliant

- RoHS Compliant

2 Applications

- Fiber Optical Instrument

- Fiber Sensor

3 Specifications

| Parameters | Unit | Value |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Operating Wavelength | nm | 1310, 1550 | 1064 | 980 | 850 |
| Bandwidth | nm | ±50 | ±30 | ±10 | ±10 |
| Typ. Insertion Loss @23℃ | dB | 0.30 | 0.40 | 0.7 | 0.8 |
| Insertion Loss @ 23℃ | dB | ≤0.5 | ≤0.6 | ≤0.9 | ≤1.0 |
| Typ. Extinction Ratio@ 23℃ | dB | 30 | 30 | 28 | 28 |
| Extinction Ratio @23℃ | dB | ≥28 | ≥28 | ≥25 | ≥25 |
| Fiber Type | / | PM1310or1550 SMF-28e | PM980 HI1060 | PM980 HI1060 | PM850 SM800 |
| Return loss | dB | ≥50 |  |  |  |
| Power handling | mW | ≤300 |  |  |  |
| Fiber Length | m | 1.0 ±0.05 |  |  |  |
| Connector | / | FC/APC |  |  |  |
| Operating temperature | ℃ | -5~+70 |  |  |  |
| Storage temperature | ℃ | -40 ~ +80 |  |  |  |
| Dimensions | mm | Ф5.5*L35 |  |  |  |

Above specifications are for devices without connectors.

For device with connector, IL is 0.3dB higher, RL is 5dB lower, ER is 2dB lower.

The default connector key is aligned to slow axis.

4 Mechanical Dimensions

5 Ordering information

| ILP | - | X | XX | X | X | XX | XX | XX | X | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Operating Wavelength | Package Type | Input Type | Output Type | Fiber Type | Fiber Length | Pigtail Type | Input Connector | Output Connector |
| ILP |  | 85=PM850 | ST=Φ5.5×L35 | 1= SMF-28e | 1= SMF-28e | 02=250um fiber | 10=1.0m | 02=250um fiber | 0=No | 0=No |
|  |  | 98=PM980 | XX= Customized | 2= PM1310 | 2= PM1310 | 09=0.9m tube |  | 09=900um loose tube | 1=FC/UPC | 1=FC/UPC |
|  |  | 10=PM1060 |  | 3= PM1550 | 3= PM1550 | NA=No Tube |  |  | 2=FC/APC | 2=FC/APC |
|  |  | 31=PM1310 |  | 4= PM980 | 4= PM980 |  |  |  | 3=SC/UPC | 3=SC/UPC |
|  |  | 55=PM1550 |  | 5= HI1060 | 5= HI1060 |  |  |  | 4=SC/APC | 4=SC/APC |
|  |  |  |  | 6= PM850 | 6= PM850 |  |  |  | 5=LC/UPC | 5=LC/UPC |
|  |  |  |  | 7= SM800 | 7= SM800 |  |  |  | 6=LC/APC | 6=LC/APC |

| Part Number | Product Description |
| --- | --- |
| ILP-55ST330910-22 | In-Line Polarizer Device，PM1550nm（Panda）Input&Output fiber is PM1550fiber, 0.9mm Hytrel White tube(length is 1.0m)With FC/APC Connetor（connector key is aligned to slow axis） |