4-channel wavelength division multiplexer

1. Basic Information

Device Name: 1-to-4 Wavelength Division Multiplexer

Specifications: 1271/1291/1311/1331nm

2. Main Technical Requirements and Specifications

Absolute Maximum Ratings

| Parameter | Symbol | Min | Max | Unit |
| --- | --- | --- | --- | --- |
| Operating Temperature | Top | -55 | 85 | ℃ |
| Storage Temperature | Ts | -55 | 85 | ℃ |

Performance Parameters

| Parameter | Symbol | Min | Typical | Max | Unit |
| --- | --- | --- | --- | --- | --- |
| Number of Channels | n | - | 4CH | - | - |
| Operating Wavelength | λc | - | 1271/1291/1311/1331 | - | nm |
| Channel Passband (0.5dB) | Passband | - | λc ±6.5 | - | nm |
| Insertion Loss | IL | - | - | 1.5 | dB |
| Ripple | - | - | - | 0.5 | dB |
| Isolation (Adjacent Channel) | ISO | 30 | - | - | dB |
| Isolation (Non-adjacent Channel) | ISO | 40 | - | - | dB |
| Polarization Dependent Loss | PDL | - | - | 0.3 | dB |
| Dimensions | - | - | 37.5 × 12.8 × 6.5 | - | mm |
| Fiber Type | - | - | G.657 A1 Fiber, 2.0mm armored tube | - | - |
| Fiber Length (COM Port) | - | - | 200 (excluding connector & channel mark) | - | mm |
| Fiber Length (Other Ports) | - | - | 300 (excluding connector & channel mark) | - | mm |
| Connector Type (COM Port) | - | - | ST/UPC | - | - |
| Connector Type (Other Ports) | - | - | LC/UPC | - | - |