Polarization Maintaining Isolator

Features

- Low Insertion Loss

- High Extinction Ratio and High Isolation

- Available for Slow or Fast Axis App

- High Stability and Reliability

- Applications

- EDFA

- Fiber Optical Instrument

- Fiber Laser

- Fiber Sensor

- Specifications

| Parameters Type |  | Unit | Single Grade |  | Daul Grade |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | P | A | P | A |
| Operating wavelength |  | nm | 1310,1480 ,1550 |  |  |  |
| Bandwidth |  | nm | ±20 |  |  |  |
| Peak Isolation |  | dB | 42 | 40 | 58 | 55 |
| Isolation@at 23℃ |  | dB | ≥28 | ≥26 | ≥48 | ≥45 |
| Insertion Loss@at 23℃ |  | dB | 0.4 | 0.5 | 0.5 | 0.6 |
| Insertion Loss@-5 ~70℃ |  | dB | ≤0.55 | ≤0.65 | ≤0.55 | ≤0.80 |
| Extinction Ratio | Type1:Fast axis blocked | dB | ≥20 | ≥18 | ≥20 | ≥18 |
|  | Type2:Both of axis working | dB | ≥25 | ≥23 | ≥25 | ≥23 |
| Return loss |  | dB | ≥55 |  |  |  |
| Power handling |  | mW | ≤300 |  |  |  |
| Fiber Type |  | / | Corning PM Panda fiber |  |  |  |
| Fiber Length |  | / | 1310nm :PM 1310;1550nm:PM1550; |  |  |  |
| Operating temperature |  | ℃ | -5 ~ +70 |  |  |  |
| Storage temperature |  | ℃ | -40 ~ +85 |  |  |  |
| Dimensions |  | mm | φ5.5 × L35(P1) (only for bare fiber or 900um loose) |  |  |  |
|  |  |  | L90*W20*H10（ABS）(P2) (only for 3mm or 2mm cable) |  |  |  |

*Above specifications are for devices without the connectors.

*For devices with connectors, IL will be 0.3dB higher, RL will be 5dB lower, and ER will be 2dB lower.

*The PM fiber and the connector key are aligned to the slow axis, fast axis is blocked

- Package Dimension

- Ordering information

| PMCIR | - | XXX | XX | XX | XX | XX | XX | - | X | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Wavelength | Type Stage | Axis | Modules type | Fiber Type | Fiber Length |  | Com connector | Port Connector |
| PMISO= PM Isolator |  | 106=1064nm | S= Single Stage | SA= slow axis | ST=Ф5.5X50 | 02=250um fiber | 10=1.0m |  | 0=None | 0=None |
|  |  | 131=1310nm | P= Daul Stage | BA= Both axis | BX=90X20 X10 | 09=0.9mm Tube | XX=Customized |  | 1=FC/UPC | 1=FC/UPC |
|  |  | 155=1550nm |  |  | XX=Customized | 20=2.0mm Tube |  |  | 2=FC/APC | 2=FC/APC |
|  |  | 165=1650nm |  |  |  | 30=3.0mm Tube |  |  | 3=SC/UPC | 3=SC/UPC |
|  |  | 148=1480nm |  |  |  | XX=Customized |  |  | 4=SC/APC | 4=SC/APC |
|  |  |  |  |  |  |  |  |  | 5=LC/UPC | 5=LC/UPC |
|  |  |  |  |  |  |  |  |  | 6=LC/APC | 6=LC/APC |
|  |  |  |  |  |  |  |  |  | X=Customized | X=Customized |

| Part Number | Product Description |
| --- | --- |
| PMISO-155PSAST10910-22(D) | PM Isolator Steel Tube Device，dual stage, Corning PM 1550nm Panda fiber with 0.9mm red loose Tube, length fiber 1m , FC/APC connector, P grade, slow axis working only |

| Part Number | Product Description |
| --- | --- |
| PMC-2215590ST10910-22(S) | 2*2 Filter PM Fiber Coupler，Splitting Ratio 90/10, Steel Tube type, PM 1550nm Panda fiber With 0.9mm Hytrel White Tube, fiber Length 1m (with connector)， connector FC/APC， connector key is aligned to slow axis. |