Gracyfiber Coarse wavelength division multiplexer (CWDM) utilizes thin film coating technology and proprietary design of non-flux metal bonding micro optics packaging. It provide low insertion loss, high channel isolation, wide pass band, low temperature sensitivity and epoxy free optical path.

Features

- Low Insertion loss and low PDL

- High channel Isolation

- Good channel-to-channel uniformity

- High Reliability and Stability

- Telcordia Compliant

- RoHS Compliant

Applications

- Broadband networks

- PON Networks

- CATV system

- CWDM system

- Specifications

| Parameters |  | Unit | 4CH | 8CH | 12CH | 18CH |
| --- | --- | --- | --- | --- | --- | --- |
| Operating Wavelength |  | nm | 1260~1620 |  |  |  |
| Channel Space |  | nm | 20 |  |  |  |
| Center Wavelength |  | nm | ITU，ITU+1 |  |  |  |
| Center Wavelength Accuracy |  | nm | ±0.8 |  |  |  |
| Pass band @-0.5dB |  | nm | ±6.5 |  |  |  |
| Port Insertion Loss |  | dB | ≤1.2 | ≤2.5 | ≤3.5 | ≤3.8 |
| Link Loss |  | dB | ≤2.0 | ≤3.2 | ≤4.0 | ≤4.3 |
| Isolation (dB) | Adjacent | dB | ≥30 |  |  |  |
|  | Non-adjacent | dB | ≥40 |  |  |  |
| Ripple |  | dB | ≤0.5 |  |  |  |
| Polarization Dependent Loss |  | dB | ≤0.2 |  |  |  |

Channel Wavelengths(ITU Grid)

| ITU-T G.694.2 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Nominal central wavelengths(nm) |  |  |  |  |
| 1 | 1270 |  | 10 | 1450 |
| 2 | 1290 |  | 11 | 1470 |
| 3 | 1310 |  | 12 | 1490 |
| 4 | 1330 |  | 13 | 1510 |
| 5 | 1350 |  | 14 | 1530 |
| 6 | 1370 |  | 15 | 1550 |
| 7 | 1390 |  | 16 | 1570 |
| 8 | 1410 |  | 17 | 1590 |
| 9 | 1430 |  | 18 | 1610 |

Structure Diagram (Refer to the below)

Mechanical Dimensions (Refer to the below)

60*40*11(mm)

100*80*10mm

120*80*18mm

140*115*18mm

Ordering information

| XX | X | XX | XX | XX | XX | - | X | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Channel No. | MUX/DEMUX Type | Initial CW | Package Type | Fiber Type | Fiber Length |  | Input Connector | Output Connector |
| 04= 02CH | M=Mux | 270= 1270 | SM=65 x45 x11 | 09=0.9mm | 05=0.5m |  | 0=No | 0=No |
|  |  |  |  | Tube |  |  |  |  |
| ………… | D= Demux | … | PS=100x80x10 |  | … |  | 1=FC/UPC | 1=FC/UPC |
| 08= 08CH | M&D= | 550=1550 | PM=120x80x18 |  | 10=1m |  | 2=FC/APC | 2=FC/APC |
|  | Mux&Demux |  |  |  |  |  |  |  |
|  |  | 551=1551 | PL=140x115 x18 |  |  |  | 3=SC/UPC | 3=SC/UPC |
|  |  |  | P1=80x58 x8 |  |  |  | 4=SC/APC | 4=SC/APC |
|  |  |  | X0= Customized |  |  |  | 5=LC/UPC | 5=LC/UPC |
|  |  |  | SM=65 x45 x11 |  |  |  | 6=LC/APC | 6=LC/APC |

| Part Number | Product Description |
| --- | --- |
| CWDM-18M271PL0905-55 | CWDM Mux Module 1271~1611nm ,18 Port , |
|  | ABS box(140*115*18mm), 0.5m with 0.9mm White Tube color, |
|  | LC/UPC connectors。 |

Technology Patent Certificate